INSTRUCTION ON DATA ENTRY TO FC_initStress-2022

Novosibirsk 2023

Table of Contents

INSTRUCTION ON DATA ENTRY TO CEMSTRESS-HYDRO	1

Control Data	3

1.1	Entering a Header. Format (a160)	3

1.2	Entering Control Data. Format (i10, 2i5)	4

1.3	Entering Time Parameters. Format (i5,i10, f10.0, 4f15.0)	5

1.4. Entering Iteration Parameters. Format (5i5,3f10.0)	6

Node Information	7

2.1	Entering Node Coordinates. Format (i10, 2i5, 2f20.0)	7

Time Functions Data	8

3.1	Time Functions Control Line. Format (2i5)	8

3.2	Entering Table Time Functions	9

Applied Load Data	10

4.1 Entering Edge Loads for 2D-Elements	10

Prescribed Nodal Displacement and Concentrated Loads	14

5.1	Nodal Displacement and Concentrated Loads Control Line. Format (2i10)	14

5.2	Entering the Prescribed Displacement. Format (i10,2i5,f10.2)	15

5.3	Entering Concentrated Loads. Format (i10,2i5,f10.2)	16

Initial Conditions	17

6.1	Initial Stress Control Line. Format (i1)	17

Elements Data. Two-Dimensional Solid Elements.	18

7.1	Element Group Control Line. Format (i5, i10, 6i5)	19

7.2	Entering Material Properties and Thickness of Group Elements.	23

7.3	Entering Group Elements Description.	25

Elements Data. 1D Elements.	27

8.1	Element Group Control Line. Format (i5, i10, 5i5)	28

8.2	Entering Material Properties and Cross-Sections of a Group of Elements	29

8.3	Entering Element Group Data	30

Section 1

Control Data

1.1	Entering a Header. Format (a160)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-160 | hed | The Grid program writes basic information into the header: Cement Sheath: plane strain, quad8: plane strain, 8-node elements) in the problem statement. |

Notes:

    - Each problem must start with a header.

1.2	Entering Control Data. Format (i10, 2i5)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-10 | numnp | total number of node points; ≥ 1; if 0 is entered, the program stops |
| (2) | 11-15 | negl | the number of linear element groups; ≥ 0 |
| (2,3) | 15-20 | negnl | the number of nonlinear element groups; ≥ 0 |

Notes:

    - The total number of nodes defines the amount of data read by the program in section 2. If numnp = 0, the program stops.

    - A group includes the elements of the same type (2D solid, truss, etc.). Elements of the first group are numerated sequentially starting from 1. The number of the first element of the following groups is equal to the total numbers of all elements of preceding groups +1. All elements of this group must have the same parameters from subsections 7.1/8.1:

      - element type (npar(1))

      - integration order (npar(10), npar(11))

The elements defined by the above mentioned parameters can be divided into more than one group; although, this is undesirable in terms of finding the best solution.

    - If nonlinear groups are not input: negnl =0, then subsection 1.4 should be skipped.

1.3	Entering Time Parameters. Format (i5,i10, f10.0, 4f15.0)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (4) | 1-5 | ivartime | time step indicator: = 0 constant time step |
|  |  |  | =1 adaptive time step |
|  | 6-15 | Nste | number of time steps (or load steps); > 0 |
| (1) | 16-25 | Tstart | problem calculation start time; =0 |
|  | 26-40 | Tfinal | problem calculation end time; > 0 |
| (2) | 41-55 | r_initial_delta_t | fraction of the end time tfinal for the initial time step dt_ini (not input parameter) |
| (3,4) | 56-70 | r_minimum_delta_t | fraction of the end time tfinal for the minimal time step dt_min (not input parameter) |
| (3,4) | 71-85 | r_maximum_delta_t | fraction of the end time tfinal for the maximal time step dt_max (not input parameter) |

Notes:

    - The time to start solving the problem, tstart, is automatically set to zero.

    - Not input initial time step parameter dt_ini = r_initial_delta_t ∙   tfinal.

    - Not input minimal and maximal time step parameters are calculated as

- dt_min = r_minimum_delta_t ∙   tfinal

- dt_max = r_maximum_delta_t ∙   tfinal

    - If  ivartime = 0, dt_min = dt_max = dt_ini are set and the parameters r_minimum_delta_t, r_maximum_delta_t can be omitted.

-

1.4. Entering Iteration Parameters. Format (5i5,3f10.0)

If nonlinear groups are not input: negnl=0, then subsection 1.4 should be skipped.

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
|  | 1-5 | Isref | number of time steps between calculation of new effective stiffness matrix; =1 by default |
|  | 6-10 | Nutmre | the number of allowable decreases in the step size at each step in time (only for a variable time step ivartime = 1) |
| (1) | 11-15 | Iequit | number of time steps between iteration process running; =1 by default |
| (1) | 16-20 | Itemax | the maximal number of allowable iterations at time step; =15 by default |
|  | 21-25 | Iteopt | the optimal number of iterations at time step (only for a variable time step ivartime = 1); =5 by default |
| (1) | 26-35 | Dtol | relative error by displacement; =0.001 by default |
| (1) | 36-45 | Ftol | relative error by stress; =0.001 by default |
| (1) | 46-55 | Etol | relative error by energy; =0.001 by default |

Notes:

    - For example, if iequit=5, itemax=12, dtol=0.001, ftol=0.002, etol=0.003 then up to 12 iterations will be run on the time steps 5, 10, 15 etc.. Convergence criteria are satisfied in case:

(1) ∥ t+∆t U (i) −t+∆t U (i−1) ∥ 2 /max∥ τ U∥ 2 ≤ dtol = 0.001, τ = ∆t,...,t,t + ∆t, here  t+∆t U (i)  is a solution on the i iteration, ∥ ∥ 2 — the Euclidean norm of a vector;

(2) ∥ t+∆t R −t+∆t F (i−1) ∥ 2 /max∥ τ R − τ −∆t F ∥ 2≤ ftol = 0.002;

(3) |( t+∆t U (i) −t+∆t U (i−1) )( t+∆t R−t+∆t F (i−1) )|/max|( t+∆t U (1) −t U)( τ R − τ −∆t F)|≤ etol = 0.003.

Section 2

Node Information

2.1	Entering Node Coordinates. Format (i10, 2i5, 2f20.0)

| Note | Position | Variable n | Value node number: |
| --- | --- | --- | --- |
| (1) | 1–10 | n | 1 ≤ n ≤ numnp |
| (2) | 11-15 | id(1,n) | code of x-axis displacement |
|  | 16-20 | id(2,n) | code of y-axis displacement |
| (3) | 31-50 | x(n) | x-coordinate |
|  | 51-70 | y(n) | y-coordinate |

Notes:

    - Node data must be defined for all numnp nodes. Node numbers can range from 1

to the total nodes number numnp. The last number entered must be equal to numnp.

    - Displacement codes can have the following values (m=1,2):

      - id(m,n)=0 — displacement along direction m is possible;

      - id(m,n)=1 — displacement along direction m equals zero.

If the node is not fixed (id(m,n)=0), displacement is possible and is determined by solving the system of equations. A directed force can be applied in this direction. Each unfixed degree of freedom is controlled by one equilibrium equation. When a degree of freedom is removed (id(m,n)=1), the corresponding equation is removed from the system of equilibrium equations. The removed degrees of freedom define fixed points (support fixation), and the applied forces associated with these degrees will not be accounted for in the program. The nodes that are used for geometrical description only (i.e., the nodes that are not assigned to elements) must have two degrees of freedom removed.

    - Two x- and y-coordinates define the position of each node.

Section 3

Time Functions Data

3.1	Time Functions Control Line. Format (2i5)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-5 | nlcur | number of time table functions t~f(t); ≥ 0 |
| (1) | 6-10 | nptm | maximum number of points used to define each of the nlcur time table functions; ≥ 0 |

Notes:

    - Time functions are entered to describe the behaviour of specific variables over time (such as load, displacement, temperature, etc.). The value of f(t) at a specific time t is calculated by linear interpolation from table t~f(t). The nptm parameter refers to the maximum number of points from [t, f(t)] that are used to describe any of the nlcur functions. Each of these functions can have fewer than nptm input points. But none of them can have more than nptm input points. Each function requires at least two points in order to be defined. Otherwise, the function cannot be interpolated.

    - If the number of time table functions nlcur = 0, subsection 3.2 must be skipped.

3.2	Entering Table Time Functions

Enter nlcur data groups in increasing order of function number.

Control line 1. Format (2i5)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-5 | k | number of time table function: 1 ≤ k ≤ nlcur |
| (2) | 6-10 | npt | number of points used to define this function: npt ≤ nptm |

Notes:

    - Time table functions must be entered in sequential order starting from number 1 and ending with nlcur.

    - The npt parameter defines the number of points ([t, f(t)] pairs) for the function.

Control line 2. Format (20f20.0)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-20 | timv(k,1) | time for the kth function at point 1 |
|  | 21-40 | rv(k,1) | value of the kth function at point 1 |
|  | 41-60 | timv(k,2) | time for the kth function at point 2 |
|  | 61-80 | rv(k,2) | value of the kth function at point 2 |
|  | ... | ... | ... |
|  | 161-180 | timv(k,5) | time for the kth function at point 5 |
|  | 181-200 | rv(k,5) | value of the kth function at point 5 |
| (2) | 1-20 | timv(k,6) | time for the kth function at point 6 |
|  | 21-40 | rv(k,6) | value of the kth function at point 6 |
|  | ... | ... | ... |

Notes:

- Enter the sequence of pairs in increasing order, i.e., timv(k,1) < timv(k,2) < timv(k,3), etc. The time value at the last point must be greater or equal to the value of time at the problem calculation end.

- In this subsection, iput as many maps as is required to define npt points ([t, f(t)] pairs), by five points on each map.

Section 4

Applied Load Data

4.1 Entering Edge Loads for 2D-Elements

Control line 1. Format (i10)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| 1 | 1-10 | npr2dc | number of conservative loads on the edges of two-dimensional elements; ≥ 0 |

Notes:

    - All conservative loads are combined into one vector Rs0 before the step-by-step integration starts.

Lines 2 and 3 contain npr2dc groups of input lines.

Control line 2. Format (i10, a1, i4, f10.0, 4i10)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-10 | m | number of the face to which the force is applied |
| (2) | 11 | iprf | the edge load type: “ ”, stress traction; “n”, normal pressure |
|  | 12-15 | itype | code of the two-dimensional solid element to which the load is applied: code 0—axisymmetric; code 1—plane strain; code 2—plane stress |
| (3) | 16-25 | thic | element thickness; 0.d0 is replaced with 1.d0 |
| (4) | 26-30 | ncur | number of a time table function describing the dependence of the given load on time: 1≤ncur≤nlcur |
| (1) | 31-35 | kg | the generation parameter used to calculate the numbers of the nodes of the missing faces (it should be given on the line with first load) 0 is replaced with 1 |
| (5) | 36-45 | nod(1) | number of the global node at point 1 of the face |
|  | 46-55 | nod(2) | number of the global node at point 2 of the face |
|  | 56-65 | nod(3) | number of the global node at point 3 of the face: set 0 if there is no internal node |
|  | 66-75 | nod(4) | number of the global node at point 4 of the face: set 0 if there is no internal node |

Notes:

- Data about faces to which an edge load data is applied are entered in increasing order of face number. If lines for faces [m + 1, m + 2, ..., m + j] are missed, then the descriptions of the missing faces are generated using the same pressure value that acts on the face m (using iprf, itype, thic, ncur, p1, p2, p3, p4, py1, py2, py3, py4 of the face m), and the numbers of the nodes of the missing faces increase by the value kg. The generation parameter kg is taken from a line of the m-th face data, and only non-zero nodes from the line increase by the value kg when the missing faces data is generated.

- The iprf code of edge load allows to specify two types of load:

  - iprf = “ “: edge load is Cauchy stress traction (px and py are components, Fig. 4.1);

  - iprf = “n“: edge load is pressure that is normal to the element face (Fig. 4.2).

- The element thickness normal to the element edge is required only in the case of plane stress element type and is ignored if itype = 0 or itype = 1.

Y

PY

NOD(2)

NOD(1)

X

Figure 4.1. Edge load is Cauchy stress traction in the global coordinate system.

Y

P>0

P1

P2

P4

P3

NOD(2)

NOD(4)

NOD(3)

NOD(1)

X

Figure 4.2. Normal pressure applied to an element face.

- The dependence of the given load on time is described by a time table function from the section 3. So that the value of the load at any time is determined by the product of the values introduced at the next map and the corresponding time function.

- An element face is defined by two nodes in the angle points nod(1) and nod(2). One or two internal nodes nod(3) and nod(4) may be present. The node nod(4) may be present only if nod(3)≠0. The sign of the normal pressure is set as follows. If an observer is looking from nod(1) at nod(2), the positive pressure corresponds to the vector that is directed from left to right from the observer's point of view (Fig. 4.2).

Control line 3. Format (8f10.0)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-10 | p1 | edge load value in nod(1) of this face in the global coordinate system along the x-axis (if iprf= “ “:) or the numerical value of normal pressure in nod(1) (if iprf=“n“) |
|  | 11-20 | p2 | - in nod(2) |
|  | 21-30 | p3 | - in nod(3) - (skip if nod(3)=0) |
|  | 31-40 | p4 | - in nod(4) - (skip if nod(4)=0) |
|  | 41-50 | py1 | edge load value in nod(1) of this face in the global coordinate system along the y-axis (skip if iprf= “n“) |
|  | 51-60 | py2 | - in nod(2) - (skip if iprf= “n“) |
|  | 61-70 | py3 | - in nod(3) - (skip if iprf= “n“ or nod(3)=0) |
|  | 71-80 | py4 | - in nod(4) - (skip if iprf= “n“ or nod(4)=0) |

Notes:

    - Edge load on an element face is determined by values p1, p2 (and optionally p3 and p4) in the angle nodes nod(1), nod(2) (and optionally nod(3) and nod(4)). Fig. 4.1 shows an example of such a load for the general case (iprf= “ “). Fig. 4.2 shows an example setting of normal pressure (iprf= “n“). Note 4 of line 2 indicates the method of specifying the normal pressure sign.

Section 5

Prescribed Nodal Displacement and Concentrated Loads

5.1	Nodal Displacement and Concentrated Loads Control Line. Format (2i10)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-10 | ndisp | number of nodes with prescribed displacement; ≥ 0 |
| (2) | 11-20 | nload | number of nodes with concentrated load; ≥ 0 |

-

Notes:

    - If ndisp=0 is entered, subsection 5.2 should be skipped.

    - If nload=0 is entered, subsection 5.3 should be skipped.

5.2	Entering the Prescribed Displacement. Format (i10,2i5,f10.2)

Enter ndips lines.

| Note | Position 1-10 | Variable node1 | Value number of the node with prescribed displacement; 1 ≤ node1 ≤ numnp |
| --- | --- | --- | --- |
|  | 11-15 | idirx | direction of the displacement: code 1 – direction of x-axis; code 2 – direction of y-axis |
|  | 16-20 | ncurx | number of the time function that is associated with the prescribed displacement |
|  | 21-30 | facx | factor of the time function |

5.3	Entering Concentrated Loads. Format (i10,2i5,f10.2)

Enter nload lines.

| Note | Position 1-10 | Variable node1 | Value number of the node in which concentrated load is given:; 1 ≤ node1 ≤ numnp |
| --- | --- | --- | --- |
|  | 11-15 | idirx | direction of action of the concentrated load: code 1 – direction of x-axis; code 2 – direction of y-axis |
|  | 16-20 | ncurx | number of the time function that is associated with the concentrated load |
|  | 21-30 | facx | factor of the time function |

Section 6

Initial Conditions

We have two options for setting initial stress.

The first is using prestress_input.txt file for Pioner execution. It is nonlinear accounted and is available for material models = 41, 42,43. In the code, the prestress from file is marked with prestress_flag.

The second is setting initial stresses σ11, σ22, σ12, σ33 as material constants only for 2d elements models=1,2,3. It is controlled in input file by the line described below.

6.1	Initial Stress Control Line. Format (i1)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| 1 | 1 | istress | code of initial stresses: code 0—initial stresses are not entered; code 1—initial stresses are entered from input file and prestress_flag=0 |

Notes:

    - Initial stresses are entered for two-dimensional elements only and may vary depending on the material; the values are entered in the material property line for two-dimensional elements.

    - If istress = 1 then prestress_flag = 0, and if nonlinearity of elements not equal to «0» then components of initial stress for every integration point reads from “input.txt” file. Initial stress read like material properties.

    - If istress = 0 and «prestress_input.txt» file is in project folder with «*.exe» file, then initial stresses read from this file, not from «input.txt».

Section 7

Elements Data. Two-Dimensional Solid Elements.

Two-dimensional elements are isoparametric quadrilateral elements with a variable number of nodes. The program supports elements with linear, quadratic, and cubic interpolation. Elements are entered in the global x,y-plane. Elements are either plane (plane strain or plane stress) or axisymmetric solid bodies. Each node of an element has two displacement degrees of freedom. When an element represents an axisymmetric body, the global y-axis is an axis of rotation. In this case, all elements must be located in the the x-positive half-plane. The edges of the elements described with three or four nodes may have a curved shape. Two-dimensional thermo- poro-elastic elements are identified by number 21.

7.1	Element Group Control Line. Format (i5, i10, 6i5)

Data in subsections 7.1–7.3 are entered sequentially for negl-1 groups of elements.

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
|  | 1-5 | npar(1) | identifier of a two-dimensional element, number 2 |
|  | 6-15 | npar(2) | number of two-dimensional elements in this group; ≥ 1 |
| (1) | 16-20 | npar(3) | indnl = index of geometric nonlinearity code 1— material nonlinearity only (MNO); code 2— total lagrangian formulation (TL) code 3— updated lagrangian formulation with Truesdell’s stress rate (ULT); code 4— updated lagrangian formulation with Hill’s stress rate (ULH); 0 is replaced with 1 |
|  | 21-25 | npar(5) | element type code: code 0—axisymmetric; code 1—plane strain; code 2—plane stress |
| (2) | 26-30 | npar(7) | maximal number of nodes used for element description: code 4—bilinear approximation; code 9—biquadratic; code 16—bicubic; 0 is replaced with 9 |
| (3) | 31-35 | npar(10) | integration order used in the Gauss quadrature in the direction r; from 0 to 4; 0 is replaced with 3 |
| (3) | 36-40 | npar(11) | integration order used in the Gauss quadrature in the direction s; from 0 to 4; 0 is replaced with 3 |
| (1,4) | 41-45 | npar(15) | material model number: code 1 — linear isotropic; code 2 — linear isotropic with tensile failure material model; code 3 — isotropic thermo-elastic model; code 4 — Hencky’s isotropic hyperelastic material model; code 41 — isotropic hypoelastic material model based on the Zaremba-Jaumann stress rate; code 42 — isotropic hypoelastic material model based on the Green-Naghdi stress rate; code 43 — isotropic hypoelastic material, based on the logarithmic stress rate; code 44 — isotropic hypoelastic material, based on the upper Oldroyd stress rate; code 45 — isotropic hypoelastic material, based on the lower Oldroyd stress rate; code 46 — isotropic hypoelastic material, based on the Hill’s stress rate; code 47 — isotropic hypoelastic material, based on the upper Truesdell stress rate; code 48 — isotropic hypoelastic material, based on the lower Truesdell stress rate; 0 is replaced with 1 |
|  | 46-50 | npar(16) | number of different material properties and element thicknesses; ≥ 1; 0 is replaced with 1 |
| (4,5) | 51-55 | npar(19) | parameter for material model 41 number: code 1 — the midpoint algorithm; code 2 — eulerian version of the absolutely objective algorithm; code 3 — eulerian version of the strictly incrementally objective algorithm; code 4 — lagrangian version of the absolutely objective algorithm; code 5 — lagrangian version of the strictly incrementally objective algorithm; 0 is replaced with 1 parameter for material model 42 number: code 1 — the midpoint algorithm; code 3 — eulerian version of the strictly incrementally objective algorithm; 0 is replaced with 1 |
|  | 56-60 | npar(21) | parameter for material model 41, 43, 46 number code1: code 1 — RA approach; code 2 — HW approach; 0 is replaced with 1 |

Notes:

    - npar(3) is used only for nonlinear group of element and specifies and determines whether the geometric nonlinearities in the solution should be taken into account. If npar(3)=1, geometric nonlinearities are not taken into account. If npar(15)=4, updated lagrangian formulation with Hill’s stress rate (ULH) or updated lagrangian formulation with Truesdell’s stress rate (ULT)  should be used. If npar(15)=41 or 42, one should use updated lagrangian formulation with Hill’s stress rate (ULH). If npar(15)=1, it is possible to use updated lagrangian formulation with Truesdell’s stress rate (ULT).

    - If npar(7) defines approximation order in this type of element, and npar(7) equals the maximal number of nodes that can be specified for this approximation. npar(7) can be set to one of three values: 4, 9, 16.

      - If npar(7)=4, then a 4-nodal element with bilinear approximation is used (Fig. 7.1).

      - If npar(7)=9, then a quadrilateral element with biquadratic approximation and a variable number of nodes from 4 to 9 is used (Fig. 7.2).

      - If npar(7)=16, then a quadrilateral element with bicubic approximation and a variable number of nodes from 4 to 16 is used (Fig. 7.3).

S

4

1

3

2

Fig. 7.1. 4-nodal element with bilinear approximation.

R

NPAR(7)=9

7

9

5

4

8

6

3

2

1

(A) parent element

(B) some parent-derived elements

Fig. 7.2. Quadrilateral element with biquadratic approximation.

    - The order of numerical integration is used to determine the integrals for calculating the stiffness matrix of elements. The integration points used to perform integration along certain coordinate (for example, r) in the local coordinate system have the following values:

| Integration order nint | Value of local coordinate |
| --- | --- |
| 1 | 0.00000 |
| 2 | ± 0.57735 |
| 3 | ± 0.77480 |
|  | 0.00000 |
| 4 | ± 0.86114 |
|  | ± 0.33998 |

Note that the integration order must be in accord with the order of approximation. For a complete 9-nodal element, a full integration order is 3*3, and for a complete 16-nodal element, it is 4*4.

S

13

14

12

16

15

11

7

9

5

10

3

6

1

2

4

R

8

(A) parent element

(B) some parent-derived elements

Fig. 7.3. Quadrilateral element with bicubic approximation.

    - Only one material model can be used in one group of elements (defined by the parameter npar (15)). Only for material molel npar(15)=41 we need to define npar(19).

    - For material molel npar(15)=41 one can use different algorithms for stress integration: the midpoint, the absolutely objective, and the strictly incrementally objective algorithms. For latest two eulerian and lagrangian versions are possible. For the eulerian version polar decomposition gives eigenvalues and eigenprojections of left stretch tensor V, for the lagrangian version polar decomposition gives eigenvalues and eigenprojections of right stretch tensor U.

7.2	Entering Material Properties and Thickness of Group Elements.

This subsection contains npar(16) groups of lines.

7.2.1	A. Material Number temp Line. Format (i5, f10.0)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-5 | n | material type number; ≥ 1 and ≤ npar(16) |
| (2) | 6-15 | thick(n) | element thickness; ≥ 0 is entered only for plane stress element type |

Notes:

      - Material types are entered in sequential order starting from 1 and ending with npar(16). Material constants and the thickness of each of the elements are identified by one of the types entered herein.

      - The element thickness should be set only for the plane stress element type problem.

7.2.2	B. Material Properties Input Lines.

Model npar (15) = 1 — linear isotropic material

Format (6f10.0)

Enter 6 values if istress=1.

| Note | Position |  | Variable | Value |
| --- | --- | --- | --- | --- |
|  | 1-10 |  | prop(1,n) | Young's modulus, E > 0 |
|  | 11-20 |  | prop(2,n) | Poisson's ratio, V ∈ (0, 12) |
|  | 21-30 |  | prop(3,n) | initial stress σ11 |
|  | 31-40 |  | prop(4,n) | initial stress σ22 |
|  | 41-50 |  | prop(5,n) | initial stress σ12 |
|  | 51-60 |  | prop(6,n) | initial stress σ33 |

Model npar (15) = 2 — linear isotropic with tensile failure material model.

Format (9f10.0)

Enter 9 values if istress=1.

| Note | Position |  | Variable | Value |
| --- | --- | --- | --- | --- |
|  | 1-10 |  | prop(1,n) | Young's modulus, E > 0 |
|  | 11-20 |  | prop(2,n) | Poisson's ratio, V ∈ (0, 12) |
|  | 21-30 |  | prop(3,n) | tensile strength, σt> 0 ( sigt) |
|  | 31-40 |  | prop(4,n) | reduction in normal direction, k1> 0 ( stifac) |
|  | 41-50 |  | prop(5,n) | reduction in tangential direction, k2> 0 (shefac) |
|  | 51-60 |  | prop(6,n) | initial stress σ11 |
|  | 61-70 |  | prop(7,n) | initial stress σ22 |
|  | 71-80 |  | prop(8,n) | initial stress σ12 |
|  | 81-90 |  | prop(9,n) | initial stress σ33 |

Model npar (15) = 3 — isotropic thermo-elastic model.

First line Format (5f10.0)

Enter 5 values if istress=1.

| Note | Position |  | Variable | Value |
| --- | --- | --- | --- | --- |
|  | 1-10 |  | prop(1,n) | reference T0 |
|  | 11-20 |  | prop(2,n) | initial stress σ11 |
|  | 21-30 |  | prop(3,n) | initial stress σ22 |
|  | 31-40 |  | prop(4,n) | initial stress σ12 |
|  | 41-50 |  | prop(5,n) | initial stress σ33 |

Second line Format (4f10.0)

Enter 4 values for each temperature T (the number of the lines is defined by imat that is prescribed to 1 in the code (npar(20)=0=>npar(20)=1; imat=npar(20)).

| Note | Position |  | Variable | Value |
| --- | --- | --- | --- | --- |
|  | 1-10 |  | prop(1,n) | temperature T |
|  | 11-20 |  | prop(2,n) | Young's modulus, E > 0 |
|  | 21-30 |  | prop(3,n) | Poisson's ratio, V ∈ (0, 12) |
|  | 31-40 |  | prop(4,n) | coefficient of thermal expansion α |

Model npar (15) = 4, 41, 42, 43 — Hencky’s isotropic hyperelastic material model or isotropic hypoelastic material models based on Green-Naghdi/ Zaremba-Jaumann / logarithmic stress rate

Format (2f10.0)

Enter 2 values.

| Note | Position |  | Variable | Value |
| --- | --- | --- | --- | --- |
|  | 1-10 |  | prop(1,n) | Young's modulus, E > 0 |
|  | 11-20 |  | prop(2,n) | Poisson's ratio, V ∈ (0, 12) |

7.3	Entering Group Elements Description.

Enter npar(2) elements in increasing order starting from 1. Each element occupies two lines.

Input line 1. Format (2i10)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| 1 | 1-10 | m | number of the two-dimensional solid element; ≥ 1 and ≤ npar(2) |
| 2 | 11-20 | mtyp | material type and thickness type assigned to this element; ≥ 1 and ≤ npar(16) |

Notes:

    - Element data is entered in order of increasing element number. Elements in groups are sequentially numbered.

    - Material types and thickness types are set in subsection 8.2.

Input line 2. Format (16i10)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-10 | nod(1) | global node number at point 1 |
|  | 11-20 | nod(2) | global node number at point 2 |
|  | 21-30 | nod(3) | global node number at point 3 |
|  | 31-40 | nod(4) |  |
|  | 41-50 | nod(5) |  |
|  | 51-60 | nod(6) |  |
|  | 61-70 | nod(7) |  |
|  | 71-80 | nod(8) |  |
|  | 81-90 | nod(9) |  |
|  | 91-100 | nod(10) |  |
|  | 101-110 | nod(11) |  |
|  | 111-120 | nod(12) |  |
|  | 121-130 | nod(13) |  |
|  | 131-140 | nod(14) |  |
|  | 141-150 | nod(15) |  |
|  | 151-160 | nod(16) | global node number at point 16 |

Notes:

- nod(k) corresponds to the global number nod for a local number k. If nod=0, then the node is skipped; if nod≥1, then the node is present. The values of nod(1), nod(2), nod(3), nod(4) never equal zero. If npar(7)=9 (see note 1 in subsection 8.1) and nod(9)≠0, then all other nodes must not be equal to zero. If npar(7)=16, then ether all or none of the central nodes (k = 13, 14, 15, 16) equal zero. If these nodes are not equal to zero, then all other nodes must not equal zero. Consider, for example, a 6-nodal element shown in Fig. 8.4:

Node 5 is not used

S

Node 7 is not used

3

6

2

1

8

R

Fig. 8.4. Example of a 6-nodal element.

This element should belong to the group, where either npar(7)=9 or npar(7)=16. In the first case,

nod(k) = [****0*0*0],

and in the second case,

nod(k) = [****0*0*00000000].

Section 8

Elements Data. 1D Elements.

One-dimensional elements include truss elements. The elements of space trusses (cable elements) are straight-line and have two nodes on its ends. The elements have hinges on its ends and arbitrary orientation in the global coordinate system (x, y). These elements are subject to axial forces only and have two degrees of freedom in each node.

8.1	Element Group Control Line. Format (i5, i10, 5i5)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
|  | 1-5 | npar(1) | enter 1—indicates a group of one-dimensional elements |
|  | 6-15 | npar(2) | total number of elements in this group; ≥ 1 |
|  | 16-20 | npar(5) | enter 0 |
| (1) | 21-25 | npar(7) | enter 2— indicates two-node truss elements; this parameter defines the order of approximation |
|  | 26-30 | npar(10) | enter 1 |
|  | 31-35 | npar(11) | enter 1 |
|  | 36-40 | npar(16) | number of different material types and cross-sections; ≥ 0 0 is replaced with 1 |

Notes:

- If npar(7) defines approximation order in this type of element, and npar(7) equals the maximal number of nodes that can be specified for this approximation. npar(7) can be set to the value 2. For two-nodal elements of the trusses (npar(7) = 2), stiffness matrices are calculated by explicit formulas without numerical integration.

8.2	Entering Material Properties and Cross-Sections of a Group of Elements

Enter npar(16) data groups.

Input line 1. Format (i5, f10.0)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
| (1) | 1-5 | n | material type number: 1 ≤ n ≤ npar(16) |
|  | 6-15 | area(n) | cross-sectional area of group elements for this material |

Note:

    - Material property types are entered in sequential order starting from 1 and ending with npar(16).

Input line 2. Format (f10.0)

Model 1. Linear-Elastic Material

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
|  | 1-10 | prop(1,n) | Young's modulus E of the material |

8.3	Entering Element Group Data

Enter npar(2) elements in increasing order.

Format (i10, i5, 2i10)

| Note | Position | Variable | Value |
| --- | --- | --- | --- |
|  | 1-10 | n | sequence number of a one-dimensional element: 1 ≤ n ≤ npar(2) |
|  | 11-15 | mtyp | type of material and cross-section assigned to the element: 1 ≤ mtyp ≤ npar(16) |
| (1) | 16-25 | nod(1) | global node number at local point 1 |
| (1) | 26-35 | nod(2) | global node number at local point 2 |

Notes:

- The nod(i) parameter corresponds to the global number nod for a local number i. For a two-node truss element (npar(7)=2), two nodes nod(1) and nod(2) are specified.